Electronics Fundamentals – laboratory

- Diode characteristics

1. Rectifier diode.

Using the laboratory board and multimeters set the measurement circuit as shown below:

Measure the values of the diode current and voltage for 10 different values of the supply voltage Vr (CAUTION! Do not exceed the 20V range of the voltage supply). Then reverse the supply voltage and measure the current and diode voltage for another 10 values of the supply voltage (do not exceed 15V of the supply). Value of Vr can be adjusted with P6 potentiometer.

Draw the diode characteristics I-V (current versus voltage).

2. Zener diode.

Rewire the laboratory board to obtain the circuits as shown below:

Repeat the procedure from the previous part. Draw the characteristics.

In both cases Vr switch should be switched on (up) to provide power to the circuit.

Electronics Fundamentals – laboratory

- Rectifiers

1. Half-wave rectifiers.

Using the laboratory board set the measurement circuit as shown below:

observe the input (J8-I9) and output (I8-I9) signal of the rectifier diode on the oscilloscope

  - with switches W7 and W8 opened

  - with switch W8 opened and W7 closed

  - with both switches closed.

Write your remarks and conclusions in the report.

2. Full-wave rectifiers

Rewire the laboratory board to obtain the circuits as shown below:

Observe the input (J7-I7) and output (J6-I6) signal of the rectifier bridge on the oscilloscope

  - with switches W5 and W6 opened

  - with switch W6 opened and W5 closed

  - with both switches closed.

Write your remarks and conclusions in the report.

In both cases Vr switch should be switched on (up) to provide power to the circuit.

- MOS transistor characteristics

1. N-channel transistor.

Use the circuit as shown below (be careful when connecting Z2, Z3, Z5 and Z6 pins).

  - Set VG to 3V. Measure the drain current ID for the following values of VD: 0V (F1 disconnected from Z2 and grounded), 1,5V, 2V, 3V, 5V, 7V, 10V. Repeat the procedure for VG set to 0V (A5 disconnected from Z5 and grounded), 0,8V, 1,5V, 5V.

  - Set VD to 3V. Measure the drain current ID for the following values of VG: 0V (A5 disconnected from Z5 and grounded), 1,5V, 2V, 3V, 5V, 7V. Repeat the procedure for VD set to 0V (F1 disconnected from Z2 and grounded), 1,5V, 5V.

NOTE: the value of the VD voltage can be regulated using P7 potentiometer; value of the VG can be changed by using P6 potentiometer.

Using the OpenOffice spreadsheet draw the graphs of ID as a function of VD and VG.

2. P-channel transistor.

Use the circuit as shown below.

  - Set VG to 3V. Measure the drain current ID for the following values of VD: 0V (F1 disconnected from Z3 and grounded), 1,5V, 2V, 3V, 5V, 7V, 10V. Repeat the procedure for VG set to 0V (A5 disconnected from Z6 and grounded), 0,8V, 1,5V, 5V.

  - Set VD to 3V. Measure the drain current ID for the following values of VG: 0V (A5 disconnected from Z6 and grounded), 1,5V, 2V, 3V, 5V, 7V. Repeat the procedure for VD set to 0V (F1 disconnected from Z3 and grounded), 1,5V, 5V.

Using the OpenOffice spreadsheet draw the graphs of ID as a function of VD and VG (remember, that this time the voltage sources are reversed).

In both cases Vr switch should be switched on (up) to provide power to the circuit.

- Operational amplifiers

Vs power switch should be switched on (up) to provide the power to the circuit. In exercise 3, also Vr should be switched on (up).

1. Inverting amplifier.

Using the laboratory board set the measurement circuit as shown below:

observe output (E1-E2) signal of the inverting amplifier on the oscilloscope

  - with switches W1 and W2 opened

  - with switch W1 opened and W2 closed

  - with both switches closed.

NOTE: Use the sinusoidal input signal (connect Z15 to Z16). Set the amplitude of the input signal to 2V (use P2) and frequency to about 1kHz (using P3). Use P4 to obtain duty cycle about 50%.

Write your remarks and conclusions in the report.

Repeat the whole procedure with triangle input signal (connect Z14 instead of Z15 to Z16)

2. Integrator.

Using the laboratory board set the measurement circuit as shown below:

observe the output (E7-F7) signal of the integrator on the oscilloscope

  - for the input signal frequency about 1kHz

  - for the input signal frequency about 10kHz

Use sinusoidal and triangle input signals with amplitude equal to 2V. Repeat the procedure with rectangular input signal (connect Z13 to Z16).

Write your remarks and conclusions in the report.

3. Summing amplifier.

Use the laboratory board set the measurement circuit as shown below:

Connect Z8 and Z6 (to have common ground level for input signals). Using P6 set the output voltage Z5-Z6 to about 3V and set the generator to obtain 1V 2kHz triangle output signal Z17-Z11 (connect Z14 to Z16; use P2 to change the amplitude of signal and P3 for adjusting the frequency).

Observe output (D2-D3) signal of the amplifier on the oscilloscope

  - with Z17 connected to C1 and Z5 to C2;

  - with Z17 connected to C1 and Z5 to C3;

  - with Z17 connected to C3 and Z5 to C2.

Write your remarks and conclusions in the report.

4. Differential amplifier

Rewire the laboratory board to obtain the measurement circuit as shown below:

Connect Z8 and Z6 (to have common ground level for input signals). Using P6 set the output voltage Z5-Z6 to about 1V and set the generator to obtain 2V 2kHz triangle output signal Z17-Z11 (connect Z14 to Z16; use P2 to change the amplitude of signal and P3 for adjusting the frequency). Connect Z5 to C5

Observe the output (D7-E5) signal of the integrator on the oscilloscope. Repeat the procedure with rectangular input signal (connect Z13 to Z16).

Write your remarks and conclusions in the report.

- CMOS logic gates

Using the integrated circuit shown below (built into the laboratory board), supplied with 5V DC,

- Determine the transfer characteristics (Uout/Uin) of a CMOS inwerter (input voltage range: 0-5V); the output voltage value should be measured for at least 10 values of Uin.

- Build the 2-input CMOS NAND gate. Determine its transfer characteristics (Uout/Uin1) for input voltage range of 0-5V

    - with the other input connected to ground (Uin2=0V – logical „0”)

    - with the other input connected to supply voltage (Uin2=5V – logical „1”)

- Repeat the procedure from exercise 2 for CMOS NOR gate.

The list of connections made and the plots of the obtained characteristics should be included into the laboratory report.

CAUTION: Built-in regulated supply voltage should be used to perform the exercises. The supply terminal of the evaluated chip should be connected to the Z2 connector and ground terminal to Z3 connector; the supply voltage can then be set using P7 potentiometer. The input voltage for the gates should be taken from the Z5 connector and regulated using P6 potentiometer (the Z6 connector should be connected to the ground).

To obtain the 0V, the input of the gate should be disconnected from Z5 connector and connected to ground (Z3 or Z6).

Be careful not to connect Z5 with Z6 and Z2 with Z3!

- Introduction to PSpice A/D simulation software

Before starting using of the PSpice A/D simulation software, one should get acquainted with “PSpice A/D Reference Manual”. The book can be obtained from the website www.dmcs.pl/~piotrowi/elfu.htm.

To open the PSpice program, open My Computer, go to the Y disk drive (programs on shrek), change the directory to Y:\msim8\bin and run pspicead.exe. To simulate the .CIR file, open the file in the program File menu. .CIR files can be written with the TextEdit program from the same directory.

      - DC analysis

The .DC command performs a linear, logarithmic, or nested DC sweep analysis on the circuit. The DC sweep analysis calculates the circuit’s bias point over a range of values for sweep variable name.

        - Read the description of .DC analysis (the Commands section) and of the MOSFET transistor device (the Analog Devices section) in the pdf file mentioned above.

        - Perform the DC analysis of the NMOS transistor, using the following .CIR file (the first line of the script must be the title of the file):

MOS output characteristics

M1 2 1 0 0 MOD1

VDD 2 0 dc 5

VG 1 0 dc 2.5

.MODEL MOD1 NMOS VTO=-1V NSUB=1E15 UO=550

.DC VDD 0 5 0.1 VG 0 5 1

.PROBE Id(M1)

.END

If the Probe program doesn’t run automatically, run it from the PSpice File menu (Run Probe command). Run command Add from the Trace menu and select Id(M1).

        - The obtained charts are the output characteristics of the transistor. Change the script to obtain the transfer characteristics of the transistor.

- AC analysis

The .AC command calculates the frequency response of a circuit over a range of frequencies.

              - Read the description of .AC analysis in the pdf file.

              - Perform the AC analysis of the low-pass filter, using the following .CIR file:

Low-pass filter

R1 1 2 1K

C1 2 0 10U

VIN 1 0 AC 1

.AC DEC 20 1 100K

.PROBE V(2)

.END

              - Basing on the circuit above, design the 2nd order low-pass and high-pass filter with the same 3dB frequency.

- TRAN analysis

The .TRAN command causes a transient analysis to be performed on the circuit and specifies the time period for the analysis.

        - Read the description of .TRAN analysis and the description of Independent Voltage Sources (the Analog Devices section) in the pdf file.

        - Perform the transient analysis of the low-pass filter, using the .CIR file from point 2. Replace the .AC analysis command with the following .TRAN command:

.TRAN 1U 1M

Input voltage source should be now described as follows:

VIN 1 0 PULSE(0 5 250U 1U 1U 500U 1M)

- Simulations with PSpice A/D

            - Parametric analysis

              - Read the description of .STEP analysis and the description of .PARAM statement (the Commands section) in the “PSpice A/D Reference Manual” file.

              - Perform the dc analysis of the CMOS inverter from the picture on the left for the following parameters of the channel: (l=4u, w=4u), (l=2u, w=4u), (l=4u, w=2u). RL value should be 1meg, V1 value should be 5V. Draw the output voltage for VIN between 0 and 5V with the step of 0,1V.

              - Perform the dc analysis of the CMOS inverter from the picture on the left for 5 different values of the load resistor RL = 100, 1k, 10k, 100k, 1meg. Set length of the MOS channels to 2u and the width to 4u. Draw the output voltage for VIN between 0 and 5V with the step of 0,1V.

Use the following models of MOS transistors:

.MODEL N NMOS LEVEL=2

+VTO=0.75 KP=60E-6 GAMMA=0.75 PB=0.91 VMAX=6E4

+CGSO=2.94E-10 CGDO=2.94E-10 CGBO=1.92E-10

+CJ=2.6E-4 MJ=0.5 CJSW=1.17E-10 MJSW=0.33

+JS=1E-16 TOX=3.5E-8 NSUB=1.65E16 XJ=0.3E-6 LD=0.22E-6

+UO=780 UCRIT=0.9E5 UEXP=0.16

.MODEL P PMOS LEVEL=2

+VTO=0.75 KP=3E-5 GAMMA=0.75 PB=0.88 VMAX=1E6

+CGSO=5.88E-10 CGDO=5.88E-10 CGBO=1.92E-10

+CJ=3.1E-4 MJ=0.5 CJSW=2.52E-10 MJSW=0.33

+JS=1E-16 TOX=3.5E-8 NSUB=6.9E15 XJ=0.6E-6 LD=0.47E-6

+UO=255 UCRIT=5.8E4 UEXP=0.42

Write your remarks and conclusions in the report. Attach the CIR files and the graphs.

- Operational amplifiers

        - Perform the transient analysis of an integrating amplifier based on the TL082 operational amplifier (use model included below) 1k resistor and 1nF capacitor, with rectangular and triangular signal with 1kHz frequency on the input. Use ±12V power supply for the amplifier and 100k resistor as a load.

        - From AC analysis of the circuit above find the 3dB frequency of the amplifier.

        - Repeat analyses from points a and b for differentiating amplifier.

.subckt TL082 1 2 3 4 5

*connections: non-inverting input, inverting input,

*positive power supply, negative power supply, output

c1 11 12 2.41e-12

c2 6 7 18e-12

css 10 99 5.4e-12

dc 5 53 dy

de 54 5 dy

dlp 90 91 dx

dln 92 90 dx

dp 4 3 dx

egnd 99 0 poly(2),(3,0),(4,0) 0 .5 .5

fb 7 99 poly(5) vb vc ve vlp vln 0 3.467e6 –1e3 1e3 3e6 –3e6

ga 6 0 11 12 339.3e-6

gcm 0 6 10 99 17.01e-9

iss 10 4 dc 234e-6

hlim 90 0 vlim 1k

j1 11 2 10 jx

j2 12 1 10 jx

r2 6 9 100e3

rd1 3 11 2.947e3

rd2 3 12 2.947e3

ro1 8 5 50

ro2 7 99 170

rp 3 4 20e3

rss 10 99 854.7e3

vb 9 0 dc 0

vc 3 53 dc 1.5

ve 54 4 dc 1.5

vlim 7 8 dc 0

vlp 91 0 dc 50

vln 0 92 dc 50

.model dx D(ls=800e-18 Rs=1)

.model dy D(ls=800e-18 Rs=1m Cjo=10p)

.model jx NJF(ls=2.5e-12 Beta=984.2e-6 Vto=-1)

.ends

- DC-DC converter

  - Perform the transient analysis of the circuit show on the picture on the right (V2 is the PULSE type source). MOS switching frequency should be 10kHz with duty cycle of 50% and the time of analysis should be 100ms. Observe the output voltage and current.

  - Perform the similar analysis for the following values of the MOS switching frequency: 100Hz, 1kHz, 20kHz, 100kHz (with duty cycle of 50%).

  - Perform the similar analysis for the MOS switching frequency equal to 10kHz for different values of the duty cycle: 10%, 25%, 50%, 75%.

Use the following models of the NMOS transistor and diode:

.model IRF350 NMOS(Level=3

+Gamma=0 Delta=0 Eta=0 Theta=0 Kappa=0.2 Vmax=0 Xj=0

+Tox=100n Uo=600 Phi=.6 Rs=9.448m Kp=20.75u W=.73 L=2u Vto=3.234

+Rd=.2206 Rds=1.778MEG Cbd=2.206n Pb=.8 Mj=.5 Fc=.5 Cgso=9.529n

+Cgdo=178.5p Rg=3.531 Is=19.74p N=1 Tt=700n)

.MODEL D1N4148 D(IS=2.682N N=1.836 RS=.5664 IKF=44.17M XTI=3

+EG=1.11 CJO=4P M=.3333 VJ=.5 FC=.5 ISR=1.565N NR=2 BV=100

+IBV=100U TT=11.54N)

Write your remarks and conclusions in the report. Attach the graphs. CIR files should be placed on your account (disc O).