68000 Assembler

by Paul McKee

User's Manual

Table of Contents

1.       Introduction .............................  2

2.       Source Code Format .......................  3

2.1       Source Line Format.......................  3

2.1.1       Label Field............................  3

2.1.2       Operation Field........................  3

2.1.3       Operand Field..........................  3

2.1.4       Comment Field..........................  4

2.2       Symbols..................................  4

2.3       Expressions..............................  4

2.3.1       Operands in Expressions................  4

2.3.1.1       Decimal Numbers......................  4

2.3.1.2       Hexadecimal Numbers..................  4

2.3.1.3       Binary Numbers.......................  5

2.3.1.4       Octal Numbers........................  5

2.3.1.5       ASCII Constants......................  5

2.3.2       Operators in Expressions...............  5

2.4       Addressing Mode Specifications...........  6

3.       Assembly Details .........................  7

3.1       Branch Instructions......................  7

3.2       MOVEM Instruction........................  7

3.3       Quick Instructions (MOVEQ, ADDQ, SUBQ)...  8

4.       Assembler Directives .....................  9

4.1       ORG - Set Origin.........................  9

4.2       Symbol Definition Directives.............  9

4.2.1       EQU - Equate Symbol....................  9

4.2.2       SET - Set Symbol.......................  9

4.2.3       REG - Register List Symbol............. 10

4.3       Data Storage Directives.................. 10

4.3.1       DC - Define Constant................... 10

4.3.2       DCB - Define Constant Block............ 11

4.3.3       DS - Define Storage.................... 12

4.4       END - End of Source File................. 13

4.5       INCLUDE - directive...................... 13

5.       Usage .................................... 14

5.1       Command Line............................. 14

5.2       Listing File Format...................... 14

5.3       Object Code File Format.................. 15

1

2

1. Introduction

The program described here, 68000 Assembler, is a basic two-

pass assembler for the 68000 and 68010 microprocessors.  It

supports the complete instruction set of both processors as well

as a modest but capable set of assembler directives. The program

produces formatted listing files as well as object code files in

S-record format.

The program was written in VAX-11 C by Paul McKee during the

fall semester, 1986.  The program should be portable (with some

changes) to any C language implementation that supports 32-bit

integers.

3

2. Source Code Format

2.1  Source Line Format

The input to the assembler is a file containing instruc

tions, assembler directives, and comments. Each line of the file

may be up to 256 characters long. It is recommended, however,

that the source lines be no longer that 80 characters, as this

will guarantee that the lines of the listing file do not exceed

132 characters in length.  The assembler treats uppercase and

lowercase identically.

Each line of the source code consists of the following

fields:

LABEL  OPERATION    OPERAND,OPERAND,...  COMMENT

For example,

LOOP   MOVE.L       (A0)+,(A1)+          Sample source line

The fields may be separated by any combination of spaces and

tabs. Except for the comment field and quoted strings, there must

be no spaces or tabs within a field.

2.1.1  Label Field

Legal labels follow the rules for forming symbol names

described in section 2.2.  Labels may be distinguished in one of

two ways:  (1) They may begin in column 1, or (2) they may end in

a colon, which does not become part of the label but simply

serves to mark its end.  A line may consist of a label alone.

When a label is encountered in the source code, it is defined to

have a value equal to the current location counter.  This symbol

may be used elsewhere is the program to refer to that location.

2.1.2  Operation Field

The operation field specifies the instruction that is to be

assembled or the assembler directive that is to be performed.  A

size code (.B, .W, .L, or .S) may be appended to the operation

code if allowed, to specify Byte, Word, Long, or Short opera

tions, respectively.  The operation field must not begin in the

column 1, because the operation would be confused with a label.

2.1.3  Operand Field

The operand field may or may not be required, depending on

the instruction or directive being used.  If present, the field

consists of one or more comma-separated items with no intervening

spaces or tabs. (There may be spaces or tabs within an item, but

only within quoted strings.)

4

2.1.4  Comment Field

The comment field usually consists of everything on a source

line after the operand field.  No special character is needed to

introduce the comment, and it may contain any characters desired.

A comment may also be inserted in the source file in another

way:  An asterisk ("*") at the beginning of the line or after the

label field will cause the rest of the line to be ignored, i.e.,

treated as a comment.

2.2  Symbols

Symbols appear in the source code as labels, constants, and

operands.  The first character of a symbol must be either a

letter (A-Z) or a period (".").  The remaining characters may be

letters, dollar signs ("$"), periods ("."), or underscores("_").

A symbol may be of any length, but only the first 8 characters

are significant.  Remember that capitalization is ignored, so

symbols which are capitalized differently are really the same.

2.3  Expressions

An expression may be used in the source program anywhere a

number is called for.  An expression consists of one or more

operands (numbers or symbols), combined with unary or binary

operators.  These components are described below.  The value of

the expression and intermediate values are always computed to 32

bits, with no account being made of any overflow that may occur.

(Division by zero, however, will cause an error.)

2.3.1  Operands in Expressions

An operand in an expression is either a symbol or one of the

following sorts of constants.

2.3.1.1  Decimal Numbers

A decimal number consists of a sequence of decimal digits

(0-9) of any length.  A warning will be generated if the value of

the number cannot be represented in 32 bits.

2.3.1.2  Hexadecimal Numbers

A hexadecimal number consists of a dollar sign ("$") fol

lowed by a sequence of hexadecimal digits (0-9 and A-F) of any

length.  A warning will be generated if the value of the number

cannot be represented in 32 bits.

5

2.3.1.3  Binary Numbers

A binary number consists of a percent sign ("%") followed by

a sequence of binary digits (0 and 1) of any length.  A warning

will be generated if the number consists of more that 32 digits.

2.3.1.4  Octal Numbers

An octal number consists of a commercial at sign ("@")

followed by a sequence of octal digits (0-7) of any length.  A

warning will be generated if the value of the number cannot be

represented in 32 bits.

2.3.1.5  ASCII Constants

An ASCII constant consists of one to four ASCII characters

enclosed in single quote marks.  If it is desired to put a single

quote mark inside an ASCII constant, then two consecutive single

quotes may be used to represent one such character.

If the ASCII constant consists of one character, then it

will be placed in the bottom byte of the 32 bit value; two

characters will be placed in the bottom word, with the first

character in the higher-order position.  If four characters are

used, then all four bytes will contain characters, with the first

in the highest-order location.  However, if three characters are

used, then they will be placed in the three  highest-order  bytes

of the 32-bit value, with 0 in the low byte (this is to accom

modate the high-byte-first addressing used on the 68000).

Note that ASCII constants in expressions are different from

strings in DC directives, as the latter may be of any length.

2.3.2  Operators in Expressions

The operators allowed in expressions are shown in the fol

lowing table, in order of decreasing precedence. Within each

group, the operators are evaluated in left-to-right order (except

for group 2, which is evaluated right-to-left).

Operators in Expressions

1.  ()  Parenthesized subexpressions

2.  -   Unary minus (two's complement)

~   Bitwise not (one's complement)

3.  <<  Shift left

>>  Shift right

4.  &   Bitwise and

!   Bitwise or

5.  *   Multiplication

/   Integer division

\   Modulus (x\y produces the remainder of x divided by y)

6.  +   Addition

-   Subtraction

6

2.4  Addressing Mode Specifications

The 68000 and 68010 provide 14 general addressing modes.

The formats used to specify these modes in assembly language

programs are listed in the table below.  The following symbols

are used to describe the operand formats:

Dn     = Data Register

An     = Address Register (SP may used instead of A7)

Xn     = Data or Address register

.s     = Index register size code (either .W or .L, .W will

be assumed if omitted)

<ex8>  = Expression that evaluates to an 8-bit value (may be

empty, in which case 0 will be used)

<ex16> = Expression that evaluates to a 16-bit value (may be

empty, in which case 0 will be used)

<ex>   = Any expression

PC     = Program Counter

Addressing Mode Specifications

Mode                                             Assembler Format

---------------------------------------------    ----------------

Data Register Direct                             Dn

Address Register Direct                          An

Address Register Indirect                        (An)

Address Register Indirect with Predecrement      -(An)

Address Register Indirect with Postincrement     (An)+

Address Register Indirect with Displacement      <ex16>(An)

Address Register Indirect with Index             <ex8>(An,Xn.s)

Absolute Short or Long (chosen by assembler)     <ex>

Program Counter with Displacement                <ex16>(PC)

Program Counter with Index                       <ex8>(PC,Xn.s)

Immediate                                        #<ex>

In addition to the general addressing modes, the following

register names may be used as operands in certain instructions

(e.g., MOVEC or EORI to CCR):

SR   = Status Register

CCR  = Condition Code Register

USP  = User Stack Pointer

VBR  = Vector Base Register (68010)

SFC  = Source Function Code Register (68010)

DFC  = Destination Function Code Register (68010)

7

3.  Assembly Details

3.1  Branch Instructions

The branch instructions (Bcc, BRA, and BSR) are unique in

that they can take a ".S" size code.  This suffix directs the

assembler to assemble these as short branch instructions, i.e.,

one-word instructions with a range to -128 to +127 bytes.  If the

".S" size code is used, and the destination is actually outside

this range, then the assembler will print an error message.  If

the ".L" size code is used, the assembler will use a long branch,

which is a two-word instruction with a range of -32768 to +32767

bytes.  If neither size code is specified, then the assembler

will use a short branch if possible (the branch destination must

be known on the first pass to be within the short branch range);

otherwise it will use long branch.

3.2  MOVEM Instruction

The MOVEM instruction, which is used for saving and restor

ing sets of registers, has one the following two forms:

MOVEM   <register_list>,<effective_address>

MOVEM   <effective_address>,<register_list>

The register list may be an explicit register list of the form

described in Section 4.2.3.  On the other hand, if a particular

set of registers is to be saved and restored repeatedly, the REG

directive (Section 4.2.3) can be used to define a register list

symbol that specifies the registers.  For example, if the regis

ter list symbol WORKSET is defined as follows:

WORKSET  REG     A0-A4/D1/D2

then the following instructions will perform the same function:

MOVEM.L  WORKSET,-(SP)

MOVEM.L  A0-A4/D1/D2,-(SP)

If a register list symbol is used, it must be defined before it

appears in any MOVEM instructions.

8

3.3  Quick Instructions (MOVEQ, ADDQ, SUBQ)

The MOVE, ADD, and SUB instructions have one-word "quick"

variations which can be used certain addressing modes and operand

values.  The assembler will use these faster variations automat

ically when possible, or they may be specified explicitly by

writing the mnemonic as MOVEQ, ADDQ, or SUBQ.

The MOVEQ instruction may be used for moving an immediate

value in the range -128 to +127 into a data register.  The assembler

will assemble a MOVE.L #<value>,Dn as a MOVEQ if the value is

known on the first pass.

The ADDQ (SUBQ) instruction adds (subtracts) an immediate

value from 1 to 8 to (from) any alterable destination.  The

assembler will use the quick form if the value is known on the

first pass to be in the range 1 to 8.

9

4.  Assembler Directives

4.1  ORG - Set Origin

The assembler maintains a 32-bit location counter, whose

value is initially zero and which is incremented by some amount

whenever an instruction is assembled or a data storage directive

is carried out.  The value of this location counter may be set

with the ORG directive.  This is typically done at the start of a

program and at appropriate places within it.  The format of the

ORG directive is

<label> ORG     <expression>

where <expression> is an expression containing no forward refer

ences, i.e., its value must be known on the first pass at the

point where the ORG directive appears.  An error will result if

an attempt is made to set the location counter to an odd value;

in this case the location counter will be set to the specified

value plus one.  The <label> is optional and, if present, the

specified symbol will be set to the  new  value of the location

counter.

4.2  Symbol Definition Directives

4.2.1  EQU - Equate Symbol

The equate directive is used to define symbols whose value

will not change within the program.  The format of this directive

is

<label> EQU     <expression>

where <expression> is an expression containing no forward refer

ences, i.e., its value must be known on the first pass at the

point where the EQU directive appears. The <label> must be speci

fied, since it tells what symbol is being defined.  If <label> is

omitted, an error will result.  If an attempt is made to redefine

a symbol that was defined with EQU, either as a label or using

any symbol definition directive, an error message will be printed.

4.2.2  SET - Set Symbol

The SET directive is similar in function and format to the

equate directive, with one important difference:  symbols defined

using SET may be redefined later using another SET directive (but

not using a EQU or REG directive). The format of this directive

is

<label> SET     <expression>

10

4.2.3  REG - Register Range

Register ranges consist of lists of registers separated by

slashes ("/").  Each register range may be either a single

register ("An" or "Dn") or a range of registers ("An-Am" or

"Dn-Dm"), which denotes all the registers between the two

registers listed (they may be given in either order).  For exam

ple, the following register list specifies that D0, D1, D2, D3,

D7, A1, A2, and A3 are to be saved (or restored):

D3-D0/D7/A1-A3

The registers and ranges may be specified in any order.  The same

format for register lists may be used with the MOVEM instruction

directly.  In order to avoid confusion, it is best to avoid

specifying a range that includes both an address register and a

data register, although the assembler will not treat this as an

error.

4.3  Data Storage Directives

4.3.1  DC - Define Constant

The define constant directive is used to store strings and

lists of constants in memory.  The format of a DC directive is

<label> DC.<size>  <item>,<item>,...

The label will be defined to equal the address of the start of

the list of data.  The size code specifies that a list of bytes

(.B), words (.W), or longwords (.L) is being defined; if omitted,

word size is used.

A list of items follows the directive; each item may be an

expression or a string. If an item is an expression, the expres

sion is evaluated and stored as the size indicated, i.e., a byte,

a word, or a longword.  An error is generated if the value will

not fit as either a signed or unsigned value in the specified

size.  If an item is a string, delimited by single quotes, then

the string will be stored in successive entities of the size

specified; if words or longwords are being generated, and the

string does not fit into an whole number of words or longwords,

then the string will be padded with zeros at the end to make a

whole number of words or longwords.  Strings and expressions may

intermixed in a single DC directive.

If words (DC.W) or longwords (DC.L) are being generated,

then the start of the list of constants will be aligned on a word

boundary by increasing the location counter by one, if necessary.

This is not performed for DC.B directives, so that strings of

bytes may be contiguous in memory.  If an instruction follows a

DC.B directive, the assembler will automatically adjust the loca

tion counter (if necessary) to place the instruction on a word

boundary.

11

An example of a DC directive that defines a null-terminated

string:

TEXT    DC.B    'DC Example',$0D,$0A,0

This directive results in the following data at location TEXT:

44 43 20 45 78 61 6D 70 6C 65 0D 0A 00  (hexadecimal)

4.3.2  DCB - Define Constant Block

The define constant block directive generates a block of

bytes, words, or longwords that are all initialized to the same

value by the assembler.  The format of the directive is

<label>  DCB.<size>  <length>,<value>

The label will be defined to equal the address of the start of

the block.  The size code specifies that a block of bytes (.B),

words (.W), or longwords (.L) is being set up; if omitted, word

size is used.

The length argument is an expression that tells the number

of bytes, words, or longwords that are to be in the block.  This

value must be known on the first pass at the point where the DCB

directive appears, and it must be non-negative.  The value argu

ment is an expression whose value is to be placed in each data

item in the block; it needn't be known on the first pass.  An

warning message will be printed if the value will not fit (as a

signed or unsigned number) in the data size selected.

12

If word or longword size is selected, then the start of the

block will be placed on a word boundary by increasing the loca

tion counter by one, if necessary.  If an instruction follows a

DCB.B directive, the assembler will automatically adjust the

location counter (if necessary) to place the instruction on a

word boundary.

4.3.3  DS - Define Storage

The define storage directive generates an uninitialized

block of bytes, words, or longwords.  The format of the directive

is

<label>  DS.<size>  <length>

The label will be defined to equal the address of the start of

the block.  The size code specifies that a block of bytes (.B),

words (.W), or longwords (.L) is being set up; if omitted, word

size is used.

The length argument is an expression that tells the number

of bytes, words, or longwords that are to be in the block.  This

value must be known on the first pass at the point where the DCB

directive appears, and it must be non-negative.  The effect of

the DS directive is basically to increase the value of the loca

tion counter by <length> times one (if DS.B is used), two (if

DS.W is used), or four (if DS.L is used)

If word or longword size is selected, then the start of the

block will be placed on a word boundary by increasing the loca

tion counter by one, if necessary.  Thus, DS.W 0 can be used to

force the location counter to be aligned on a word boundary

without allocating any space.  However, if an instruction follows

a DS.B directive, the assembler will automatically adjust the

location counter (if necessary) to align the instruction on a

word boundary.

13

4.4  END - End of Source File

The end directive is used to mark the end of the source

file.  It is purely optional.  The format is simply

END

The assembler will ignore anything in the source file after the

END directive.

4.5 INCLUDE directive

The include directive is used to make another file seem as

if it is a part of the source file.  When the include directive is

found, processing stops on the source file and starts immediately

on the file named in the directive.  This has the effect of making

it as if the contents of the included file had been contained in

the source file at the point of the include directive.  Processing

continues on the include file until the end of file is reached.

Processing then resumes on the original file at the line immediately

after the include directive. The syntax is:

INCLUDE        function.asm

Note that if the same label is defined both in the source file

and the include file an error will occur.

14

5.  Usage

5.1  Command Line

The 68000 Assembler is run by typing a command line of the

following form:

68kasm <options> <filename>

The options are a string of letters, preceded by a dash,

which alter the behavior of the assembler. The following option

letters are allowed:

c = Show all the Constants produced by DC directives

l = Produce a Listing file

n = Produce No object code file

a = Produce long word absolute addresses only

If these options are not specified, the defaults are to show only

one line of data from a DC directive, to produce no listing, and

to produce an object file.

The filename is the name, including directory specifica

tions, of the file to be assembled.  No default file extension is

applied.  The names of the listing and object code files, if

generated, are constructed by using the source file name with an

extension of ".lis" (for the listing) or ".h68" (for the object

file); the output files are always placed in the user's default

directory.

The program will print "68000 Assembler by PGM" when it

begins work.  If any errors occur, the program will print "ERROR

in line X" or "WARNING in line X" (this information is also

placed in the listing file).  Upon conclusion, it will print the

number of errors encountered and warnings generated during the

assembly.

If there is an error in the command line, e.g., if no file

name is specified, then the assembler will print a brief usage

summary and abort.

5.2  Listing File Format

The assembler produces a listing file which shows the source

code alongside the the object code produced by the assembler.  A

typical listing file line might appear as follows (not to scale):

0000102E  22D8           200  LOOP  MOVE.L (A0)+,(A1)+    Sample

The eight digit hexadecimal number at the left is the assembler's

location counter; the generated instruction word, $22D8, is

placed at that address.  The next number is the source file line

number, and the remainder of the line simply repeats the source

line.  Remember that if the source lines are no longer than 80

columns, then the listing file lines will not exceed 132 columns.

15

If an error is encountered or a warning is generated for a

given source line, then in the listing that line will be followed

by a line that describes the error.  At the end of the listing,

the program prints the total number of errors and warnings.

There is only limited space to list the object code in this

format.  There is sufficient space for the longest possible

instruction, but the DC directive poses a problem, since it may

generate, e.g., dozens of longwords from a single source line.

The assembler's -c command line option controls the assembler's

actions when the object code exceeds the space available on one

line.  If -c is not specified, then the assembler will print only

one listing line with an ellipsis ("...") at the end of the

object code field, indicating that some of the data produced by

the directive was omitted from the listing.  If -c is included on

the command line, then the assembler will use as many source

lines as are needed to print all the data produced by the direc

tive.  Each line after the first will contain only the location

counter and the object code field (the source line is not re

peated).

5.3  Object Code File Format

The 68000 Assembler produces an object code output file in

S-record format.  The object file name is the source file name,

with the extension changed to ".h68".  The object file and the

listing file are always placed in the user's default directory.

The S-record format is designed to allow files containing

any data to be interchanged in text file format with checksum

error detection.  The format of these files will not be described

here, but the following technical information will be provided:

The first line of the object file is an S0 (header) record and

the last line is an S9 (termination) record.  The lines in be

tween are S1, S2, or S3 records, depending on the whether the

address of the first byte in a record requires 2, 3, or 4 bytes

to be represented.  No record is longer than 80 characters.